IO-Link Communication Standard

| Md Wasib Kamal Nirjon Electronic Engineering Hamm-Lippstadt University of Applied Sciences Lippstadt, Germany md-wasib-kamal.nirjon@stud.hshl.de | Md Ratul Ahmed Alvi Electronic Engineering Hamm-Lippstadt University of Applied Sciences Lippstadt, Germany md-ratul-ahmed.alvi@stud.hshl.de |
| --- | --- |

Abstract

This paper presents an initial-stage analysis of the IO-Link communication standard (IEC 61131-9) as a point-to-point interface for industrial sensors and actuators. We examine the protocol architecture, parameterization mechanisms, diagnostic capabilities, and a brief comparison with traditional analog and digital I/O. IO-Link is positioned as a complementary last-meter digitalization layer within broader industrial automation stacks (EtherCAT, PROFINET, OPC UA). Key metrics include latency, throughput, jitter, and cycle time.

Keywords: IO-Link, IEC 61131-9, industrial communication, sensor interface, Industry 4.0, parameterization

1.  Introduction

Modern industrial automation requires smarter, bidirectional communication at the sensor and actuator level. Traditional interfaces — analog 4–20 mA current loops and binary digital I/O — carry only a single process value with no device identity, no diagnostics, and no remote parameterization [1]. IO-Link (IEC 61131-9, 2013) addresses this gap by providing a universal three-wire point-to-point link for the last meter between master and field device [2].

The paper is organized as follows: Section 2 reviews related fieldbus evolution. Section 3 describes the IO-Link architecture. Section 4 evaluates key metrics and compares IO-Link with traditional I/O. Section 5 concludes with an outlook.

2.  Literature Review

Industrial communication has evolved through three generations. Classic fieldbuses (PROFIBUS, MODBUS) introduced deterministic communication but remained proprietary [3]. Industrial Ethernet (PROFINET, EtherCAT) brought higher bandwidth and real-time capability [4]. Neither generation standardized sensor-level parameterization or diagnostics. IO-Link (IEC 61131-9:2013) was introduced to fill this last-meter gap over existing three-wire cabling [1][5].

Existing literature covers IO-Link in discrete manufacturing [6] and condition monitoring [7]. This work focuses on a performance comparison between IO-Link and traditional I/O using standard KPIs.

3.  System Architecture and Protocol Stack

3.1  Physical Layer

IO-Link uses a three-wire interface (L+, L−, C/Q), maximum cable length of 20 m, over standard unshielded M8/M12 connectors compatible with existing automation wiring. Backward compatibility with standard I/O (SIO mode) is preserved.

3.2  Communication Modes

The master polls up to eight device ports per cycle. Three speeds are defined:

- COM1: 4.8 kbaud — legacy and low-speed devices

- COM2: 38.4 kbaud — standard automation sensors

- COM3: 230.4 kbaud — high-speed devices

Each M-sequence frame carries cyclic process data, acyclic ISDU (Indexed Service Data Unit) for parameterization, and event/diagnostic codes within a single exchange [2].

3.3  Parameterization and Diagnostics

Each device is described by an IODD (IO Device Description) XML file. On device swap, the master automatically verifies device identity and restores parameters. Diagnostic events are transmitted in real time to the PLC or SCADA system, enabling predictive maintenance without additional wiring.

4.  Evaluation

4.1  Key Performance Indicators

- Latency: approximately 2–5 ms per polling cycle

- Throughput: up to 230.4 kbps at COM3

- Jitter: below 1 µs at COM3

- Cycle time: 0.4 ms to 40 ms depending on device class

- Determinism: master-driven polling, bounded timing

4.2  Comparison with Traditional I/O

- Analog I/O (4–20 mA): one process value, no device identity, EMI-susceptible, manual calibration required

- Digital I/O (binary): on/off signal only, no parameterization, no event reporting

- IO-Link: bidirectional, multi-parameter, plug-and-play swap via IODD, real-time diagnostics, no extra wiring cost

IO-Link complements — not replaces — controller-level protocols such as EtherCAT. For sub-millisecond determinism at the PLC level, IO-Link master polling overhead must be accounted for in system design [6].

4.3  Application Areas

- Smart factory edge nodes: pressure, temperature, and flow sensors with embedded intelligence

- Condition monitoring: vibration sensors transmitting diagnostics for predictive maintenance

- Collaborative robotics: end-of-arm tooling reconfiguration via ISDU without production stops

- Process automation: remote parameter adjustment and automatic device replacement

5.  Conclusions

IO-Link provides a standardized, last-meter point-to-point link for industrial sensors and actuators, delivering parameterization, diagnostics, and bidirectional process data over existing three-wire cabling. Compared to traditional I/O, it offers richer device data, plug-and-play device management, and real-time event reporting without additional wiring cost.

Future work will evaluate IO-Link Wireless and integration with OPC UA information models for Industry 4.0 vertical data pipelines.

References

[1]  International Electrotechnical Commission, IEC 61131-9: Programmable Controllers — Part 9: Single-drop digital communication interface for small sensors and actuators (SDCI), 2013.

[2]  IO-Link Consortium, IO-Link Interface and System Specification, Version 1.1.3, 2019. [Online]. Available: https://io-link.com

[3]  M. Felser, "Real-time ethernet — industry prospective," Proceedings of the IEEE, vol. 93, no. 6, pp. 1118–1129, 2005.

[4]  M. Wollschlaeger, T. Sauter, and J. Jasperneite, "The future of industrial communication," IEEE Industrial Electronics Magazine, vol. 11, no. 1, pp. 17–27, 2017.

[5]  R. Zurawski, Ed., Industrial Communication Technology Handbook, 2nd ed. Boca Raton, FL: CRC Press, 2014.

[6]  A. Bony, P. Pedreiras, and L. Almeida, "Performance characterization of IO-Link," in Proc. IEEE ICIT, 2019, pp. 1–6.

[7]  R. Pigan and M. Metter, Automating with PROFINET, 3rd ed. Erlangen: Publicis, 2017.

[8]  B. Galloway and G. P. Hancke, "Introduction to industrial control networks," IEEE Communications Surveys & Tutorials, vol. 15, no. 2, pp. 860–880, 2013.

[9]  J. Moyne and D. Tilbury, "The emergence of industrial control networks," Proceedings of the IEEE, vol. 95, no. 1, pp. 29–47, 2007.

[10]  D. Loy, D. Dietrich, and H.-J. Schweinzer, Open Control Networks: LonWorks/EIA 709 Technology. Norwell, MA: Kluwer, 2001.